# VU memory layout: Sprite Pipeline

### Notes:

For a better understanding of this example, knowledge on RenderWare PowerPipe architecture and PS2 architecture are required.

You can refer to the RenderWare Graphics API Reference and the “Getting Started with PowerPipe” document, both installed in your RenderWare Graphic SDK directory.

The PPVU1 example is also a good start for using PowerPipe and VU code within the PowerPipe Architecture.

As a side reading the pdf  “An Introduction to PlayStation® 2” by James Russell, from SCEE Technology Group (located on the Sony web site) is also recommended.

### Clusters Definitions

|  |  | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Input Cluster Stride | 4 | XYZ | RGBA | HW | U1V1U2V2 |  |
| Output Cluster Stride | 5 | STQ1 | RGBA | XYZW1 | STQ2 | XYZW2 |

### Number of vertex per batch

The pipeline uses 2 Input Buffers and 2 Output buffers plus one giftag for each output buffer

= (SymbStaticDataStart-2)/(2*SymbStrideOfInputCluster + 2*SymbStrideOfOutputCluster)

= 54

### Size of Input buffer

= SymbPointCount*SymbStrideOfInputCluster

= 216

### Size of Output buffer

= SymbPointCount*SymbStrideOfOutputCluster

= 270

### Buffers Addresses

Address of input buffer 1	= Start of Memory

= 0

Address of input buffer 2	= Address of input buffer 1 + Size of Input buffer

Called VifOffset	= 216

Address of output buffer 1	= Address of input buffer 2 + Size of Input buffer

= 432

Address of output buffer 2	= Address of output buffer 1 + Size of Output buffer + 1

= 703

The 2 sets of buffers are used alternatively until the completion of the Batch.

This scheme avoids memory access overlapping during pipeline execution.

For example: for 270 Sprites, there will be 270/54=5 Batches sent to the VU1.

| Step | Input Buffer 1 | Input Buffer 2 | Output Buffer 1 | Output Buffer 2 |
| --- | --- | --- | --- | --- |
| 1 | DMA In | NOP | NOP | NOP |
| 2 | Input Read | DMA In | Output Written | NOP |
| 3 | DMA In | Input Read | XGKICK | Output Written |
| 4 | Input Read | DMA In | Output Written | XGKICK |
| 5 | DMA In | Input Read | XGKICK | Output Written |
| 6 | Input Read | NOP | Output Written | XGKICK |
| 7 | NOP | NOP | XGKICK | NOP |